What kinds of the thermocouples are used?

The table below has seven widely used thermocouples. There are about ten thermocouples listed in the international (IEC) and/or national standards, like ANSI of the U. S. A., DIN of Germany and JIS of Japan. The listed thermocouples are recognized as reliable in research and application, among possible combinations of dissimilar metals.

The national standards have adopted most of the thermocouples listed in the IEC standard. Table below contains thermocouples typically used in international trade.

A thermocouple is identified by the Type, for example Type B or B thermocouple. The positive and negative wires identify the polarity of the thermocouple. When the hot junction is higher in temperature than the cold junction, a positive emf appears on the positive wire.

A thermocouple can be used above the recommended temperature range, up to the upper temperature limit. Above the recommended range the life of the thermocouple becomes short, due to oxidation, crystallization of the wires and other reasons. In some applications, the only possible, economical way of temperature measurement is to use a thermocouple near the upper temperature limit, replacing it frequently.

There are new thermocouples, like Type N (nicrosil/nisil) thermocouple, which are expected to have a longer life than Type K thermocouples in an oxidizing atmosphere. DIN includes Types U and L. In the U. S., Type W is used in very high temperature (up to 2300°C or 3700°F).

The standards carry the temperature-emf tables of each thermocouple, referenced to 0°C (32°F). In other words, it is the emf when the cold junction is at 0°C.

| TYPE | WIRE TERMINALS |  | ACCURACY CLASS | DIAMETER (mm) | RECOMMENDED TEMP. RANGE °C (°F) | UPPER TEMP. LIMIT °C (°F) |
| --- | --- | --- | --- | --- | --- | --- |
|  | POSITIVE WIRE | NEGATIVE WIRE |  |  |  |  |
| B | Platinum -rhodium (rhodium30%) | Platinum -rhodium (rhodium30%) | 0.5 | 0.50 | 1500 (2732) | 1700 (3092) |
| R | Platinum -rhodium (rhodium30%) | Platinum | 0.25 | 0.50 | 1400 (2552) | 1600 (2912) |
| S | Platinum -rhodium (rhodium13%) | Platinum | 0.25 | 0.50 | 1400 (2552) | 1600 (2912) |
| N | Alloy of mostly nickel,chromium and silicon | Alloy of mostly nickel and silicon | --- | 0.65 1.00 1.60 2.30 3.20 | 850 (1562) 950 (1742) 1050 (1922) 1100 (2012) 1200 (2192) | 900(652) 1000(1832) 1100(2012) 1150(2102) 1250(2282) |
| K | Alloy of mostly nickel and chromium | Alloy of mostly nickel | 0.4 0.75 1.5 | 0.65 1.00 1.60 2.30 3.20 | 650 (1202) 750 (1382) 850 (1562) 900 (1652) 1000 (1832) | 850 (1562) 950 (1742) 1050 (1922) 1100 (2012) 1200 (2192) |
| E | Alloy of mostly nickel and chromium | Alloy of mostly copper and nickel | 0.4 0.75 1.5 | 0.65 1.00 1.60 2.30 3.20 | 450 (842) 500 (932) 550 (1022) 600 (1112) 700 (1292) | 500 (932) 550 (1022) 650 (1202) 750 (1382) 800 (1472) |
| J | Iron | Alloy of mostly copper and nickel | 0.4 0.75 1.5 | 0.65 1.00 1.60 2.30 3.20 | 400 (752) 450 (842) 500 (932) 550 (1022) 600 (1112) | 500 (932) 550 (1022) 650 (1202) 750 (1382) 750 (1382) |
| T | Copper | Alloy of mostly copper and nickel | 0.4 0.75 1.5 | 0.32 0.65 1.00 1.60 | 200 (392) 200 (392) 250 (482) 300 (572) | 250 (482) 250 (482) 300 (572) 350 (662) |

