Do Current-to-voltage Transducers Give You Strange Output Readings?

Why does the output fluctuate even though the input signal terminals are not connected?

Why do I get output readings even though the positive (+) terminal on the input signal side has been disconnected?

Theoretically speaking, it is correct to assume that when the input terminals are not connected there is no output. This holds true for current-to-voltage transducers but not always for voltage input units, as explained below.

Current-to-voltage transducers have a resistor connected at the socket; rack-mounted transducers have them installed internally. When the input terminals are disconnected, this resistor provides the necessary resistance for a 0V output.

DC voltage input units all have high input impedance ratings. This means that external noise can effect the unit to a point where output changes notably, or even quite substantially.

M-System has flexible solutions to meet your specific application and requirements. Consult our Signal Conditioners Data Library. 